ECE 755

DNN Project Report

Quality Metric

| Area (mm2) | Max Freq. (MHz) | Min Latency (ns) | Power (mW) | Energy (pJ) | EDAP (pJ ns-mm2) |
| --- | --- | --- | --- | --- | --- |
| 420.68 | 1.028 | 1945.5 | 0.714 | 1389 | 1.14 x 109 |

Initial design consisted of 2 hidden layers with the Relu comparison and 2 output. The valid signals are available to indicate when the output is ready after the calculation.

Optimizations that went under in further milestones are

- Tighten bit width of registers with range analysis. Considering the worst-case scenario width of registers were chosen in order to account for these scenarios but not expanding them enough so that bits are wasted later.

- Pipelining the design into 2 stages for timing closure in synthesis for lower clock period and achieving better frequency.

- The design was cleaned in order to remove redundant register and FSMs.

- Ready signal was also pipelined for latency tracking.